Normative part

| Nr | Parameter | Description | Comments |
| --- | --- | --- | --- |
| 1 | Radiocommunication Service | Mobile Service |  |
| 2 | Application | MFCN | MFCN in the 24.25-27.5 GHz band shall not be used for connectivity from base stations to terminals on-board UAV. Only communications for connectivity from terminals on-board UAV to base stations is authorised (see decides 7 in ECC Decision (18)06). |
| 3 | Frequency band | 24.25-27.5 GHz | CEPT administrations shall make available by the end of 2020 at least 1 GHz for MFCN in this band, subject to market demand (see decides 2 in ECC Decision (18)06). |
| 4 | Channelling | The frequency arrangement is a TDD arrangement with a block size of 200 MHz; This block size could be adjusted to narrower blocks (multiples of 50 MHz) adjacent to other users, to allow full use of spectrum, if required; If blocks need to be offset to accommodate other uses, this shift to be done in 10 MHz steps. |  |
| 5 | Modulation / Occupied bandwidth | Not specified | Technology neutral approach |
| 6 | Direction / Separation | TDD: not applicable |  |
| 7 | Transmit power / Power density | Least Restrictive Technical Conditions (LRTC) applicable to the MFCN systems are specified in Annex 2 of ECC Decision (18)06: Base station: - Table 2: MFCN BS transitional region requirements for coexistence between MFCN networks in adjacent blocks (assuming synchronised operation) - Table 3: MFCN BS baseline requirements for coexistence with MFCN networks in other (non-adjacent) blocks in the band (assuming synchronised operation) - Table 4: MFCN BS additional baseline requirement: maximum emissions into the 23.6-24.0 GHz band - Table 5: Conditions applying to the elevation of the main beam from 5G AAS outdoor base stations Terminal station: - Table 6: MFCN terminal station maximum emissions into the 23.6-24.0 GHz band | The maximum in-block radiated power may be set in the licensing provisions (individual Right of Use). |
| 8 | Channel access and occupation rules | Tables 2 and 3 in ECC Decision (18)06 are defined assuming synchronised operation. Administrations may define appropriate mitigation measures to be applied in case of unsynchronised or semi-synchronised operations, taking into account, if available, an ECC Report on a toolbox for coexistence of MFCN in unsynchronised or semi-synchronised operations. Alternatively, administrations may further develop and use an appropriate block edge mask at national level. |  |
| 9 | Authorisation regime | Individual autorisation | The technical conditions related to coexistence with other services attached to ECC Decision (18)06 have been developed on the assumption of an individual authorisation framework; any other assumption on the authorisation framework, such as general authorisation or a combined individual/general authorisation regime may require different and/or supplementary technical conditions (see considering j) in ECC Decision (18)06). |
| 10 | Additional essential requirements according to Art. 3.3 of RE Directive | None |  |
| 11 | Frequency planning assumptions | Appropriate provisions are needed in the authorisation for MFCN to define precisely how to safeguard in a proportionate way the use of existing EESS/SRS receiving earth stations and the possibility for future earth station deployments in the 25.5-27 GHz frequency band (see considering o) in ECC Decision (18)06). Appropriate provisions are needed in the authorisation for MFCN to define precisely how to safeguard in a proportionate way the use of existing FSS transmitting earth stations and the possibility for future earth station deployments in the 24.65-25.25 GHz frequency band (see considering p) in ECC Decision (18)06). The protection of Radio Astronomy Service (RAS) observations in the 23.6-24.0 GHz band will require the implementation of suitable separation distances between RAS stations and MFCN transmitters on a case-by-case basis. |  |

Informative Part

| Nr | Parameter | Description | Comments |
| --- | --- | --- | --- |
| 12 | Planned changes | A regular assessment of the evolution of MFCN system characteristics, including network deployments, in a timeline consistent with the 5 years review process of the Decision, or sooner if necessary, will provide additional confidence that these LRTC ensure adequate protection of other services, in particular space services (see considering p) in ECC Decision (18)06). |  |
| 13 | Reference | EC Decision : under development Harmonised Standards: TBD CEPT: ECC Decision (18)06 (corrected 26 October 2018) |  |
| 14 | Notification number |  |  |
| 15 | Remarks |  |  |